# Akshmen Sutharshan – 20576162

# Drawing Robot software planning document

# Outline of the Problem to be Solved

(Maximum 1 page) (I have mentioned where I will be using functions through small indicators [])

The purpose of this project is to code a program that can send G code commands to a drawing robot that writes text inputted by an individual. This will be achieved by processing 2 files. First the program will read the singlestrokefont.txt file to understand the instructions to draw each character. This will then be stored in a structure that contains each character and its movement commands [F1]. Then it will ask the user to input their desired “height” for all the characters, and it will only allow a valid number between 4 and 10 (mm) inclusive [F2]. The provided “height” will then be divided by a factor of 18 to scale all G code coordinates from the singlestrokefont.txt file to achieve this size [F3].

A user inputted text file will be read by the program, allowing the robot to bring up the necessary commands to write this text [F4]. For the specific character, the movement data from the structure containing the font file characters will be read [F5]. This information will then be converted into G code commands which can be executed by the robot [F6]. The program will ensure that there is an offset between adjacent characters as to stop the robot writing them over each other [F7]. Line breaks will also be managed, when the 100mm width limit has been reached, by resetting the X coordinate to the left-hand margin and moving the Y coordinate 5mm below the completed line [F8].

Before the robot begins to draw, it will be prompted with initialisation commands [F9]. These consist of M3, which turns on the spindle so the drawing arm can move, and S0, to raise the pen and prevent drawing when configuring the robot to its original position. Serial communication will be used to transmit all G code commands to an Arduino. This micro controller will respond with “ok” after receiving commands [F10], and a final command line will ensure that the robot returns to its original coordinates with the pen raised [F11].

# Key Data Items

(SSF file = SingleStrokeFont.txt file)

| Name | Data type | Rationale |
| --- | --- | --- |
| FontCharacterList | Struct SSFfont [128] | This array of structures will store movement commands for each 128 ASCII characters from the SingleStrokeFont.txt file. This supports [F1] with loading the font and [F5] for accessing the movement commands of a specific character. |
| Height | Float | This variable will hold the input value requested by the user, between 4 to 10 mm inclusive. A float variable contains decimal positions, which can provide precision. This data is gained through [F2]. |
| ScaleFactor | Float | This variable will hold the value after the “height/18” calculation. This value is returned by [F3] and is used by [F6] to scale the coordinates of G code commands. |
| CharCurrent | Char | This char variable will hold the current character to be processed from the text file [F4]. It is used in [F5] to extract movement data into an array called MoveCommands. |
| MoveCommands | Struct MC [35] | This array of structures will store the sequence of Movement Commands needed for the current character being drawn (CharCurrent). This is the output of [F5] and is used by [F6] to convert the commands into Gcode. |
| Gcodebuf | Char [50] | This buffer will contain strings for a single G code command which the robot will execute (e.g. = G0 X0 Y0 S1000). This buffer is the output of [F6] and is used in multiple functions involving robot initialisation [F9], Arduino responses [F10] and resetting the robot back to original position [F11]. |
| Xcurrent Ycurrent | Float | These floats track the CURRENT (X, Y) coordinates of the pens drawing position. These variables support functions [F7, F8] when applying character offsets and line breaks respectively. |
| LineBreakSpacing | Const Float | This constant float stores negative 5.0 which is the value for each line break spacing. This is used by [F8] when a line break occurs. |
| MaxLineWidth | Const Float | This constant float stores 100.0 (mm) due to the maximum line width limit. This is used by [F8] to ensure the max limit has been reached resulting in a line break. |
| PENstate | Int | This variable can change between pen states (0 = raised, 1 = drawing). This is monitored to prevent duplicate S0/S1000 commands being sent. PENstate is used by [F9, F11] for the initial and final state of the pen being raised. |
| SerialPort | int | Serial communication sent to the Arduino is represented by this file descriptor. The COM port used is identified by the integer value. [F9] uses this to send initialisation commands and [F10] to transmit all G code commands which receive “ok” responses from the Arduino. |
| okBuf | Char [16] | This buffer stores “ok” responses by the Arduino during serial communication. This buffer is used by [F10] to manage “ok” response while checking for error messages. This ensures smooth serial transmission. |
| TEXTbuf | Char [100] | This buffer will store processed data from the text file. A 100-character limit will be able to store all word sequences. This will assist [F4] in storing text that is being read. |

Extend table as required

# Function Declarations

Only include functions that you will develop.

Inputs, Outputs will be declared as (i) and (o) respectively

[F1] = LoadSSFfile (const char* Filename, SSFfont FontCharacterList [ 128])

Parameters:

- (i) Filename = SingleStrokeFont.txt file path

- (o) FontCharacterList = This array will store SSFfile font structures (ASCII 0-127)

Return value = If this Is successful then this will return 128 and if it fails then return -1

[F2] = UserInputTextHeight ( void )

Parameters:

- No input or output parameters

Return value = If successful, a float measurement between 4 and 10 mm will be returned. However, it will return -1 to notify the user that there was an error and is prompted to put a number within the range provided.

[F3] = ScaleFactorCalculation ( float Height )

Parameters:

- (i) Height = This is the variable that stores the number the user inputted

- There are no output parameters

Return value = This will return the result from (Height/18) as a float

[F4] = ProcessTextFile ( const char* TextFile, SSFfont FontCharacterList [] , float ScaleFactor)

Parameters:

- (i) TextFile = This is a path to the text file containing characters to be drawn

- (i) FontCharacterList = This array will store font structures (ASCII 0-127) and is used by [F5] to    lookup movement data for characters

- (i) ScaleFactor = This variable is used by F6 to scale G code commands

Return value = If a file error ( file is not opened) occurs then -1 will be returned. 0 will be returned if all the characters in the text file is processed

[F5] = CharacterMovementsNeeded ( char CharCurrent, struct SSFfont FontCharacterList [], struct MC MoveCommands [] )

Parameters:

- (i) CharCurrent = will store the current character from the text file that will be processed

- (i) FontCharacterList = This array will store font structures and is used to lookup movement data for characters

- (o) MoveCommands = This array stores Movement Commands from CharCurrent to be converted to G code commands

Return value = If the character is found and the number of movements is stored in MoveCommands then it will return 1. If a character is not found, then it will return -1. If 0 is returned, then there is a “space”, and no movements are added to the array.

[F6] MovementToGcode (struct MC MoveCommands [], int NumOfMovement, float *Xcurrent, float *Ycurrent, float ScaleFactor, char *Gcodebuf, int SerialPort)

Parameters:

- (i) MoveCommands = This array stores Movement Command structures for the character that was processed

- (i) NumofMovement = This integer contains the number of movements in the MoveCommands array

- (i, o) Xcurrent/Ycurrent = The pointers will read the starting position of the pen and update the X, Y position of the pen after drawing the character

- (o) Gcodebuf = This buffer will contain the Gcode commands.

- (i) ScaleFactor = Used to scale the coordinates of the movement commands

- (i) SerialPort = This is required to call F10 after generating the Gcode string in Gcodebuf

Return value = If the G code string is generated and stored inside Gcodebuf then this will return 0. If the coordinates are invalid after scaling this will return -1

[F7] CharacterPositionUpdate ( float *Xcurrent, float *Ycurrent, float Xfinal, float Yfinal )

Parameters:

- (i, o) Xcurrent/Ycurrent = These pointers will read the starting position and update the position to the final position of the character that was drawn previously. This then becomes the new starting position for the character to come.

- (i) Xfinal/Yfinal = These floats will update the X,Y position to the last movement of the character that was just drawn

Return value = 0

[F8] LineBreakHandling (float *Xcurrent, float *Ycurrent, float MaxLineWidth, float LineBreakSpacing)

Parameters:

- (i, o) Xcurrent/Ycurrent = These pointers read the current X, Y position of the pen. X then changes to 0 and Y will decrement by 5

- (i) MaxLineWidth = checks the current line width against the defined maximum limit (100mm)

- (i) LineBreakSpacing = this variable is equal to negative 5 which is space between lines

Return value = 0

[F9] RobotStartUp ( int SerialPort, int *PENstate, char Gcodebuf[] )

Parameters:

- (i) SerialPort = this handles the serial connection to the Arduino so commands can be sent

- (o) PENstate = This pointer will keep track of the pen state, 0 is raised and 1 is drawing state. This will be set to 0 initially.

- (o) Gcodebuf = This buffer temporarily holds G code commands before it is sent(e.g. F1000, M3)

Return value = 0, if all commands to initialise the robot was created and carried out successfully. It will return -1 if any command was not sent successfully.

[F10] GcodetoArduino ( const char *Gcodebuf, int SerialPort,  char okBuf [] )

- (i) Gcodebuf = contains G code commands to send to Arduino

- (i) SerialPort = This handles serial communication with the Arduino

- (o) okBuf = This buffer stores Arduino responses (ok)

Return value = If “ok” is received then this will return 0 and -1 if no “ok” response is received

[F11] ReturnPenToOrigin ( int SerialPort, float *Xcurrent, float *Ycurrent, int *PENstate, char Gcodebuf [ ] )

Parameters:

- (i) SerialPort = This handles serial communication with the Arduino

- (o) Xcurrent/Ycurrent = The pointers will reset to (0,0) on the robot drawing interface

- (o) PENstate = This pointer will set the pen state to 0 (raised)

- (o) Gcodebuf = This buffer will temporarily contain the G code command to return the pen to the origin

Return value = 0

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| [F1] LoadSSFfile | (1) File loaded | Filename = SingleStrokeFont.txt | 128 (success) |
|  | (2) File does not exist | Filename = NotSingleStrokeFont.txt | Return -1 (fail) |
|  | (3) Invalid file format | Filename = Invalid.pdf | Return -1 (fail) |
| [F2] UserInputTextHeight | (1) Valid number within range | UserInput = 5 | 5.0 (success) |
|  | (2) Boundary numbers | UserInput = 4 , 10 | 4.0 , 10.0 (success) |
|  | (3) Invalid numbers out of range | UserInput = 15 | Return -1 (prompt again) (fail) |
|  | (4) Not a number | UserInput = hello, %%% | Return -1 (prompt again) (fail) |
| [F3] ScaleFactorCalculation | (1) Middle range scaling | Height = 9.0 | 0.50 |
|  | (2) Boundary scaling | Height = 4.0 | 0.22 |
| [F4] ProcessTextFile | (1) TextFile is read | TextFile = Hi.text | 0 (TEXTbuf stores “hi”) (success) |
|  | (2) TextFile does not exist | TextFile = NotExist.txt | Return -1 (fail) |
|  | (3) TextFile contains nothing | TextFile = Nothing.txt | 0 (file opens but no characters are read) (success) |
| [F5] CharacterMovements Needed | (1) Character is listed | SSFfont[128] CharCurrent = S | No. of movement commands for “S” Return 1 (success) |
|  | (2) Character is not listed | CharCurrent = $ | Return -1 (fail) |
|  | (3) Inputting a space between words | CharCurrent = (space) | No. of movements to input a space = 0 Return 0 (success) |
| [F6] MovementToGcode | (1) Movement is converted | MoveCommands = contains valid data for a letter (e.g. S). NumofMovement = (no.) of moves needed for (S.) Xcurrent/Ycurrent = 0 and ScaleFactor is user choice | 0 Gcodebuf contains strings for Gcode. (success) |
|  | (2) No movements (space) | NumOfMovement = 0 | 0 Gcodebuf does not contain anything. Xcur and Ycur update with F7 (success) |
|  | (3) Incorrect scaling | ScaleFactor = 0 | Return -1 (fail) |
| [F7] CharacterPosition Update | (1) Positioning of the next character is updated | Xcurrent = 5.0 Ycurrent = 5.0 Xfinal = 10.0 Yfinal = 5.0 | 0 Xcurrent should be 12.0 if the offset is fixed to 2mm (success) |
|  | (2) Positioning from origin | Xcurrent = 0.0 Ycurrent = 0.0 Xfinal = 8.0 Yfinal = 0.0 | 0 Xcurrent should be 10.0 if the offset is 2mm (success) |
| [F8] LineBreakHandling | (1) Line width has reached or gone over 100mm | Xcurrent = 100.0 or 104.0 Ycurrent = 0.0 MaxLineWidth = 100.0 | 0 Xcurrent resets to 0 and Ycurrent updates to negative 5.0 (success) |
|  | (2) Line width has not reached 100mm | Xcurrent = 99.9 Ycurrent = 0.0 MaxLineWidth = 100.0 | 0 Xcurrent and Ycurrent do no change (success) |
| [F9] RobotStartUp | (1) Robot starts up | SerialPort = 2 (COM 2) | 0 PENstate = 0, Gcodebuf contains startup Gcode (e.g. M3,S0) (success) |
|  | (2) Robot does not start up | SerialPort = -2 | -1 Commands are not sent through (fail) |
| [F10] GcodetoArduino | (1) “ok” response confirmed after command | Example Gcodebuf G1 X5 Y5 F1000 SerialPort = valid to a COM port on Arduino (e.g. 2) | 0 (success) |
|  | (2) “ok” response is not confirmed after command | Example Gcodebuf G1 X5 Y5 (No F) Arduino does not respond | Return -1 (fail) |
|  | (3) Serial port is invalid | Gcodebuf ( code listed above) SerialPort = -2 | Return -1 (fail) |
| [F11] ReturnPenToOrigin | (1) Return pen to the origin (0,0) | SerialPort = check with Arduino if connection is successful Xcurrent = 30.0 Ycurrent = 20.0 PENstate = 1 | 0 Xcurrent = 0.0 Ycurrent = 0.0 PENstate = 0 G code moves pen back to (0,0) (success) |
| Main () | (1) Successful completion of all code | File system works and user input is within ranges specified | Robot can draw the text successfully |
|  | (2) SSFont file is not found | SingleStrokeFont.txt file is not found | Exit after F1 fails. |
|  | (3) TextFile is not found | The users TextFile.txt is not found | Exit after F4 fails |
|  | (4) Successful Line breaks | A text that exceeds 100mm limit | Successful insertion of line break |

Extend table as required. Note that ‘Function’ includes main()

# Flowchart(s)

Included as a separate PDF

# AI Statement

Error Identification and Correction: The AI helped identify and correct inconsistencies in data types, function parameters, and logical errors in flowchart sequences (e.g., ensuring the pen was raised before moving the robot in F11).

Grammar and Clarity: The AI suggested improvements to grammar, spelling, and phrasing to make the document more professional and clearer.

Logic and Structure Refinement: While I designed the core logic, the AI helped refine the structure of function declarations and improved the completeness of the flow in flowcharts.